IRIS  —  Web UI Settings Guide

http://192.168.1.200:5000   |   Changes save instantly.  Use Persist to SD to survive reboot.

Audio tab

Controls how IRIS listens after you say the wake word.

Adult

| Setting | What it does | Default | Range |
| --- | --- | --- | --- |
| Max record time | How long IRIS waits for you to finish talking before giving up. | 10s | 1-60s |
| Wait after silence | How long IRIS waits after you stop talking before sending your words for processing. Lower = faster. | 1.5s | 0.1-10s |
| Mic sensitivity | How quiet the room must be before IRIS considers you done. Raise this in noisy rooms. | 300 | 50-5000 |

Tip: Reducing 'Wait after silence' from 1.5s to 0.7s cuts ~0.8s off every response — the single biggest latency win.

Kids Mode

Same three settings with more forgiving defaults — longer waits, quieter mic gate.

| Setting | Default |
| --- | --- |
| Max record time | 14s |
| Wait after silence | 3.5s |
| Mic sensitivity | 150 |

Wake Word tab

Controls how IRIS detects the wake word.

| Setting | What it does | Default | Range |
| --- | --- | --- | --- |
| Detection confidence | How sure IRIS needs to be before waking up. Higher = fewer false triggers, may miss quiet wake words. | 0.90 | 0.5-1.0 |
| Startup delay | Brief audio flush after wake word fires so it does not bleed into your question. Rarely needs changing. | 0.15s | 0.05-1.0s |

Voice tab

Controls how IRIS speaks. Chatterbox (on GandalfAI) is the primary voice; Piper is the offline fallback.

| Setting | What it does | Default |
| --- | --- | --- |
| Chatterbox on/off | Turn the main voice engine on or off. Off = falls back to simpler Piper voice. | On |
| Reference voice | Audio file used to clone IRIS's voice. Swap this to change how IRIS sounds. | iris_voice.wav |
| Expressiveness | 0 = flat/robotic 0.45 = dry wit (default) 1.0 = theatrical | 0.45 |

Conversation tab

Controls back-and-forth conversation behaviour.

Follow-up & Context

| Setting | What it does | Default | Range |
| --- | --- | --- | --- |
| Follow-up window | After answering, how long IRIS keeps listening for a follow-up before going back to sleep. | 2s | 1-60s |
| Kids follow-up window | Same, for kids mode. | 15s | 1-120s |
| Max follow-up turns | Back-and-forth exchanges before IRIS forgets the conversation and starts fresh. | 3 | 1-20 |
| Memory timeout | If nobody talks to IRIS for this long, it forgets the conversation context. | 5 min | 30s-1hr |

Response Length

IRIS auto-picks how long to make its answer based on the question. No restart needed.

| Tier | When IRIS uses it | Default length |
| --- | --- | --- |
| Short | Yes/no, greetings, quick facts | ~2-3 sentences |
| Medium | Explanations, how-to questions | ~4-6 sentences |
| Long | Stories, step-by-step guides, lists | ~8-12 sentences |
| Max | Tell me everything about..., essays | ~15+ sentences |
| Voice cutoff | Maximum spoken before Chatterbox cuts off | ~5-8 sentences |

Sleep tab

Controls sleep/wake behaviour and display brightness at night.  Auto-schedule: Sleep 9 PM / Wake 7:30 AM.

Mouth Display Brightness

| Setting | What it does | Default | Range |
| --- | --- | --- | --- |
| Awake brightness | Mouth display brightness during the day. | 8 | 0-15 |
| Sleep brightness | Mouth display brightness at night. Keep this very dim. | 1 | 0-15 |

'Save & Apply Now' changes brightness immediately without a restart.

Sleep LED Glow  (requires assistant restart)

| Setting | Default |
| --- | --- |
| Peak brightness | 26 (very dim by design) |
| Floor brightness | 3 |
| Pulse speed | 8s per cycle |

Lights tab  (requires assistant restart)

LED glow colour and pulse while IRIS is awake.

| Setting | Normal — cyan | Kids mode — yellow |
| --- | --- | --- |
| Peak brightness | 65 | 62 |
| Floor brightness | 3 | — |
| Pulse speed | 5s | 4s |

Gandalf AI tab

Controls which AI brain IRIS uses to answer questions.

| Setting | What it does | Default |
| --- | --- | --- |
| Adult model | AI model used for adult conversations. | iris |
| Kids model | AI model used when kids mode is active. | iris-kids |

System tab

Speaker Volume

| Setting | What it does | Default | Range |
| --- | --- | --- | --- |
| Volume slider | IRIS's speaker volume right now. Takes effect immediately. | 121 | 0-127 |
| Volume ceiling | Maximum volume that voice commands (turn it up) are allowed to reach. | 127 | 60-127 |

SD Persistence

| Status bar colour | Meaning |
| --- | --- |
| Green | Saved to SD card — safe on reboot. |
| Amber | RAM only — will be lost on reboot. Click Persist to SD. |

How a response happens

Every voice interaction follows this chain. The Bench tab shows real timings for each stage.

You say the wake word

-> brief audio flush (~0.15s)

-> IRIS records you speaking

-> you stop talking

-> IRIS waits  <-- BIGGEST TUNABLE DELAY

Audio tab: Wait after silence  (default 1.5s, try 0.7s)

-> audio sent to Whisper for speech-to-text

-> text sent to AI for an answer

-> answer spoken aloud via Chatterbox